AG LAB MANAGEMENT SAFETY

The following specifications are from the “Planning Facilities for Vocational Agriculture Departments”, Missouri Department of Elementary and Secondary Education & University of Missouri-Columbia, Ag-Ed Dept., June 1984.

- Size of Mechanics Shop:

Lamb’s Formula: 100 sq. ft. per student in shop-largest class plus Number of Mechanics times 56 sq. ft. plus 20% of this total

  - 50’ to 60’ wide by 70’ to 80’ long

  - Large door: 14’ high by 16’ wide or more

  - Arc, Mig, Tig Welding

  - Woodworking Benches

  - Tool Storage

  - Lumber and Metal Storage (225 sq. ft. + 100 additional sq. ft. for materials)

  - Ventilation

  - Wash Sink

  - Safety

    - Flammable Storage

    - Safety Areas and Safety Lines

    - Fire Extinguishers

    - First Sid Cabinet

    - Shop Storage Room

    - Acetylene and Oxygen stored, chained up-right 20’ apart unless a firewall is in-between

- Classroom:

  - Minimum of 780 sq. ft. (exclusive of storage room and laboratory space with minimum width of 26’ and minimum length of 34’)

    - 30 sq. ft. / student in largest class

  - Light reflective value of at least 60% for walls; 80% + ceilings

  - Artificial light to provide 100-foot candles at the working level in the poorest lighted areas

  - Table Vs. Desks

- Library:

  - Shelves, cabinets, bulletins, magazines, notebooks, record books

- Office/ Conference:

  - Minimum 10’ width x 14’ length

  - 75-foot candles measured at desk level

- Classroom Storage:

  - Width 5’-6’, length 8’-10’

- Greenhouse:

  - Inside Lighting

    - Surround lighting

    - Distance from other buildings

    - Walkways and tables

- American Disability Association (meets ADA)