Pleasant Hill High School

Safe Operating Procedures

MIG/GMAW Welder

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses (100% Cotton heavy denim welding shirt)

- Closed toe shoes and NO pants with “fringes”

- No rings or other jewelry

- Contain long and loose hair

- PPE´s (welding helmet, gauntlet gloves, pliers & brush)

- #10 welding helmet (May include Respirator)

Potential Hazards

- Burns and Fire Hazard

- Eye Injuries

- Electric Shock

- UV radiation to skin and eyes

- Toxic fumes

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Make sure there are no flammables around

- Say “Cover” to shield others from the flash

- Do not weld items that have flammables in them

- Set to correct polarity (wire type and application specific)

  - Self-Shielding Flux-Cored:  DC--

  - Solid Wire with protective gas:  DC+ (Set gas psi 28-30)

- Set to correct voltage & wire speed (See chart under welder panel)

- Turn on air cleaner and ventilation fan on

- Remove cell phones and electronics

- Mark hot metal with chalk

- Turn on shielding gas (unless self-shielding flux-cored welding)

Pleasant Hill High School

Safe Operating Procedures

Cold Metal Cut-Off Saw

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses and safety shield

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair

- PPE´s

- Hearing Protection

- Contain loose clothing

Potential Hazards

- Noise may cause hearing damage

- Eye Injuries from flying objects

- Sharp Edges

- Tipping

How to Use the Machine

- Do not force blade into the metal

- Use high speed for ⅛ inch or less

- Use low speed for 3/16 inch or thicker

- Use stands to support long stock

- Make sure coolant is circulating to cool blade before cutting

- Make sure metal is secure in vise before cutting

- Cut with the vise jaws less than ½ inch away

- Cut AL++ on high speed

- Keep spindles and guides lubricated

- Clean metal chips daily

Pleasant Hill High School

Safe Operating Procedures

Metal Shear

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses and safety shield

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair

- PPE´s

- Hearing protection

- Contain loose clothing

Potential Hazards

- Pinching or cutting fingers

- Noise can cause hearing damage

- Sharp edges

- Eyes injuries from flying objects

- Tipping

How to Use the Machine

- Metal must be secure under the yellow roller guard before cutting to ensure it will not spring the shear and pinch your finger

- Unplug daily to avoid cord tripping hazards

- Do not shear anything over ¼ inch thickness

- Keep fingers away from the shear and shear guard while cutting

- Secure stock while punching a hole

- Secure long stock with metal stands

- Thin metal will be cut with a board to support on the backside

to prevent jaw bending over cutting edge

Pleasant Hill High School

Safe Operating Procedures

Compound Miter Saw (Wood and/or Metal)

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses (Face Shield for metal cutting)

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair

- PPE´s (100% Cotton heavy denim welding shirt)

- Hearing protection

- Contain loose clothing

Potential Hazards

- Puncture wounds, Cuts

- Kick backs

- Sparks may burn clothes or skin (metal saw)

- Noise may cause hearing damage

- Sharp edges

- Eye injury to flying wood or metal chips

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Use a vice or a clamp to support

- Do not have your body in the path of the blade

- Secure work-piece tightly in vise jaws

- Keep hands away from cutting area

- Make sure it is turned off before leaving

- 4¨ safety margin from fingers to blade minimum

Pleasant Hill High School

Safe Operating Procedures

Pedestal/ Bench Grinder

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses and Safety shield

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair

- PPE´s

- Hearing protection

- Contain loose clothing

Potential Hazards

- Hot metal

- Noise may cause hearing damage

- Sharp edges

- Eye injuries from flying objects

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Check all guards and safety devices to ensure they are in position before starting the grinder

- Ensure wheels do not touch the work-piece and the gap between wheels and work-piece is no greater than ⅛ inch

- Do not grind on the side of the wheel

- Hold small objects firmly with plyers

- Never force the work-piece into the stone

- Do not use grinder stones less than ⅓ of its original depth (size) or one down to “manufacturer paper”

- Do not use stones that vibrate excessively (True with Wheel-dresser)

Pleasant Hill High School

Safe Operating Procedures

Radial Arm Saw

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses and Safety Shield

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair and clothing

- PPE´s

- Hearing protection

Potential Hazards

- Kick backs

- Eye injuries from flying objects

- Noise may cause hearing damage

- Cuts!

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Stock must be firmly against the fence

- Keep hands clear of the blade, 6¨ margin of safety

- Allow blade to reach a complete stop before leaving

- Do not cross arms when cutting

- Return cutting head to the rear of the table after cut

- When cutting bowed timber, place bow against table

- Always disconnect the power, prior to main adjustments

- No ripping (potential for blade to “throw” wood

- Keep guards and anti-kickback down

- Hold cutting arm until blade has stopped

Pleasant Hill High School

Safe Operating Procedures

Table Saw

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair and clothing

- PPE´s

- Hearing protection

Potential Hazards

- Dust control, Floor may get slick

- Noise may cause hearing damage

- Cuts and pinching areas

- Eye and body injuries from flying material

How to Use the Machine

- Set the blade so it extends less than ¼”  above material to be cut:

**Three Teeth Rule** no more than 3 teeth should be above stock

- Stand to one side of the line of the saw blade when it is on

- Feed material slowly

- Use the push stick when needed and when ripping

- Never leave the table saw running unattended

- Maintain 4¨ margin for safety from the blade

- Position of the stock must be controlled by the miter gauge

- Stop the saw before making any adjustments to the fence or blade

- Do not use both the miter and fence together while cutting

Pleasant Hill High School

Safe Operating Procedures

Plasma Cutter

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair

- PPE´s (No pants with fringes!)

- Hearing protection

- Respiratory protection

- Leather gloves (100% Cotton heavy denim welding shirt)

- Welding lens for #3-5 required

Potential Hazards

- Cuts

- Burns/Fire Hazard

- Shock/ Electricity

- Eye injuries from flying objects

- Noise may cause hearing damage

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Ensure machine is setup for correct amps and airflow

- Ensure that you have a good connection

- Metal coated with or containing materials that emit toxic fumes should not be heated/cut unless coating is removed from work surface

- Do not leave the plasma cutting unattended while running

- Use wood straight edge to guide cut

- Metal less than 1/8” rest tip on metal; above 1/8” thick use spacer

Pleasant Hill High School

Safe Operating Procedures

Drill Press

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair

- PPE´s

- Contain loose clothing and lanyards (ties included)

Potential Hazards

- Hot metal

- Noise may cause hearing damage

- Sharp edges

- Spinning stock

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Make sure there are no flammables around

- Allow chuck to stop before leaving the machine

- Always have PPE´s on

- Never hold metal by hand

- Never leave the key in the chuck

- Use brush to clear the surface and “swarf”

- When drilling, keep long stick to the left

- Do not use dull bits, Use cutting fluid

- Do not use gauntlet leather gloves

- Do not try to stop stock

- Use correct speed.  I.E.    200/ ½ bit = 400 RPM

Pleasant Hill High School

Safe Operating Procedures

Belt Sander

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses

- Closed toe shoes

- No rings or other jewelry

- Contain long and loose hair

- PPE´s

- Contain loose clothing

- Hearing protection

- Dust Mask

Potential Hazards

- Shock Hazards

- Slick floors from dust

- Hearing Damage

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Ensure that the material being worked is well supported

- Never rest the sander on its side while still running

- Use a clamp to hold the material secure where necessary

- Always hold with BOTH hands

- Allow the belt to reach a full speed before using

- Only make adjustments when the machine is turned off

- Allow machine to come to a complete stop before leaving

Pleasant Hill High School

Safe Operating Procedures

ARC/SMAW Welder

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses (100% Cotton heavy denim welding shirt)

- Closed toe shoes and Pants with NO “Fringes”

- No rings or other jewelry

- Contain long and loose hair

- PPE´s (May include respirator)

- Leather gauntlet gloves, Pliers, Chipping Hammer and Brush

- #10 shade lenses

Potential Hazards

- Burns/Fire Hazard

- Eye injuries from flying objects

- Toxic fumes

- Electric shock

- UV radiation to skin and eyes

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Do not lay electrode holder on the table when machine is on

- Turn air cleaner and ventilation fan on before welding

- Close curtain behind you

- Say “Cover” before striking arc to warn others to prevent arc flash

- Make sure there are no flammables around

- Do not weld items that have flammables in them

- Always have PPE´s on

- Adjust correct polarity for electrode and amps for metal thickness

- Place cell phones and electronics in safe location as damage may occur

- Figure Starting Amps by dividing electrode end thickness.  1/8” = .125

1 amp for every .001 of an inch = 125 amps for ¼’ metal thickness

Pleasant Hill High School

Safe Operating Procedures

TIG/GTAW Welder

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses (welding helmet, gauntlet gloves, pliers & brush)

- Closed toe shoes and Pants with NO “Fringes”

- No rings or other jewelry

- Contain long and loose hair

- PPE´s (Do not weld with cell phones or electronics)

- #10-12 welding helmet (100% Cotton heavy denim welding shirt)

Potential Hazards

- Burns & Fire Hazards

- Eye injuries from flying objects

- Electric shock

- UV radiation to skin and eyes

- Toxic fumes

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Make sure there are no flammables around

- Say “Cover” to shield others from flash

- Do not weld items that have flammables in them

- Set to correct Amps (according to metal thickness)

- Set to correct Polarity (AC = Aluminum,  DC—Mild and Stainless Steel)  DC+ in only used to round tungsten

- Turn on air cleaner and ventilation fan

- Remove oxidation on Aluminum with sand paper or brush

- Turn on Shielding Gas to correct psi (28-30 psi for Argon)

- Use separate brushes for each type of metal (cross-contamination)

Pleasant Hill High School

Safe Operating Procedures

Oxy-Acetylene/Fuel Equipment

Do not use this machine without permission from an instructor

Safety Requirements

- Safety exam must be passed with 100% accuracy

- Safety glasses

- Closed toe shoes

- Welding gloves

- No rings or other jewelry

- Contain long and loose hair

- PPE´s (100% Cotton heavy denim welding shirt)

- #3-5 shade shielding protection for eyes

Potential Hazards

- Burns / Fire Hazards

- Eye injuries from flying objects

- UV radiation to skin and eyes

- Toxic fumes

How to Use the Machine

- Do not use faulty equipment, if so find an instructor

- Do NOT go above 15 psi on acetylene fuel

- Drain the fuel lines and take pressures off the regulator when done

- *CDL= Close the Tank, Drain the Line & Loosen the regulator handle

- Adjust gases to thickness of what is to be welded/cut

- Welding tips 3< set pressures to 4 acetylene and 4 oxygen

- Do not weld/cut items that have flammables in them

- Turn on air cleaner and ventilation fan on

- Protect gas line hoses and concrete floor surface from molten metal

- Have catch pan to catch molten metal and clear away flammables

- Mark hot metal with chalk

- Cutting:  ¼” =4 acet./20 Oxy;         ¾”=4-5 acet./30-35 Oxy  etc.

____Your Name_____________ High School

Safe Operating Procedures

Use as Needed!

Do not use this machine without permission from an instructor\

Safety Requirements

-

Potential Hazards

-

How to Use the Machine

-